Compilation date:  10/04/2017

Revision date:  20/07/2016

# Revision No:  2

§

Section 1: Identification of the substance/mixture and of the company/undertaking

1.1. Product identifier

Product name:   Prosan Food Grade QAC Wipes, (PN100/1/2/3/4/6/10/12/18/19 & 306)

1.2. Relevant identified uses of the substance or mixture and uses advised against

Use of substance / mixture:   Sanitising Cleaning wipe.

1.3. Details of the supplier of the safety data sheet

Company name:   Maclin Sourcing Solutions  Limited

Unit A3

Risby Business Park, Newmarket Rd., Risby

Suffolk, IP33 3RU

Tel:   +44(0)1284810887

Fax:   +44(0)1284811908

Email:  info@maclingroup.co.uk

1.4. Emergency telephone number

Emergency tel:   +44(0)1284810887 (daytime)

Section 2: Hazards identification

2.1. Classification of the substance or mixture

Classification under CLP:   Eye Irrit. 2: H319;

Classification under CHIP:   -: R52/53

Most important adverse effects:. Causes serious eye irritation.

2.2. Label elements

# Label elements under CLP:

Hazard statements:   H319 Causes serious eye irritation

Signal words:   WARNING

Hazard pictograms:   GHS07: Exclamation mark

Precautionary statements:   P264: Wash hands thoroughly after handling.

P305+351+338: IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing.

P337+313: If eye irritation persists: Get medical advice/attention.

2.3. Other hazards

PBT:   This product is not identified as a PBT/vPvB  substance.

Section 3: Composition/information on ingredients

3.2. Mixtures

# Hazardous ingredients:

PROPAN-2-OL

| EINECS | CAS | CHIP Classification | CLP Classification | Percent |
| --- | --- | --- | --- | --- |
| 200-661-7 | 67-63-0 | - | Flam. Liq. 2: H225; Eye Irrit. 2: H319; STOT SE 3: H336 | 1-10% |

QUARTERNARY AMMONIUM COMPOUNDS

| 270-325-2 | 68424-85-1 | - | Acute Tox. 4: H302; Skin Corr. 1B: H314; ACUTE TOX. 4: H312 | <1% |
| --- | --- | --- | --- | --- |

Section 4: First aid measures

4.1. Description of first aid measures

Skin contact:   Remove all contaminated clothing/footwear immediately unless stuck to skin.

Wash immediately with plenty of soap and water.

Eye contact: Bathe the eye with running water for 15 minutes.

Ingestion: As the substrate is a wipe material ingestion is highly unlikely but in the event do not induce vomiting and wash mouth out with water.

Inhalation:   Consult a doctor.

4.2. Most important symptoms and effects, both acute and delayed

Skin contact:   There may be irritation and redness at the site of contact.

Eye contact: There may be pain, irritation and redness. The eyes may water profusely.

Ingestion:   There may be irritation to the throat

Inhalation:   There may be irritation of the throat .

Delayed / immediate effects:   Not applicable

4.3. Indication of any immediate medical attention and special treatment needed

Immediate / special treatment:   Eye bathing equipment should be available on the premises.

Section 5: Fire-fighting measures

5.1. Extinguishing media

Extinguishing media: Alcohol resistant foam, water spray, carbon dioxide or dry powder chemical

5.2. Special hazards arising from the substance or mixture

Exposure hazards:   In combustion emits toxic fumes.  Forms explosive air vapour mixture.

5.3. Advice for fire-fighters

Advice for fire-fighters: Wear self-contained breathing apparatus. Wear protective clothing to prevent contact with skin and eyes.

Section 6: Accidental release measures

6.1. Personal precautions, protective equipment and emergency procedures

Personal precautions: Do not attempt to take action without suitable protective clothing - see section 8 of SDS. Do not create dust. Eliminate all sources of ignition.  Prevent the build up of electrostatic charge in the immediate area.  Ensure lighting and electrical equipment are not a source of ignition.

6.2. Environmental precautions

Environmental precautions:   Do not discharge into drains or rivers.

6.3. Methods and material for containment and cleaning up

Clean-up procedures: Transfer to a closable, labelled salvage container for disposal by an appropriate method.

6.4. Reference to other sections

Reference to other sections:   Refer to section 8 of SDS. Refer to section 13 of SDS.

Section 7: Handling and storage

7.1. Precautions for safe handling

Handling requirements:   Smoking is prohibited.  Use non-sparking tools. Ensure there is sufficient ventilation of

the area.  Avoid the formation or spread of dust/mists in the air.

7.2. Conditions for safe storage, including any incompatibilities

Storage conditions:   Store in cool, well ventilated area. Keep container tightly closed.  Keep away from sources of ignition. Prevent the build up of electrostatic charge in the immediate area.  Ensure lighting and electrical equipment are not a source of ignition.

7.3. Specific end use(s)

Specific end use(s): PC35: Washing and cleaning products (including solvent based products).

Section 8: Exposure controls/personal protection

8.1. Control parameters

# Hazardous ingredients: PROPAN-2-OL

Workplace exposure limits:

Respirable dust

| State | 8 hour TWA | 15 min. STEL | 8 hour TWA | 15 min. STEL |
| --- | --- | --- | --- | --- |
| UK | 999 mg/m3 | 1250 mg/m3 | - | - |

DNEL/PNEC Values

| Hazardous ingredients: PROPAN-2-OL Type | Exposure | Value | Population | Effect |
| --- | --- | --- | --- | --- |
| DNEL | Oral | 26 mg/kg bw/day | Consumers | Systemic |
| DNEL | Dermal | 319 mg/kg bw/day | Consumers | Systemic |
| DNEL | Dermal | 888 mg/kg bw/day | Workers | Systemic |
| DNEL | Inhalation | 89 mg/m³ | Consumers | Systemic |
| DNEL | Inhalation | 500 mg/m³ | Workers | Systemic |
| PNEC | Fresh water | 140.9 mg/l | - | - |
| PNEC | Fresh water sediments | 552 mg/kg | - | - |
| PNEC | Marine water | 140.9 mg/l | - | - |
| PNEC | Marine sediments | 552 mg/kg | - | - |
| PNEC | Soil (agricultural) | 28 mg/kg | - | - |

8.2. Exposure controls

Engineering measures:   Ensure there is sufficient ventilation of the area. Ensure lighting and electrical equipment

are not a source of ignition.

Respiratory protection:   Respiratory protection not required.

Hand protection:   Nitrile gloves. Personal Protective Equipment, not required under normal use.

Eye protection:   Safety glasses. Ensure eye bath is to hand.

Skin protection:   Personal Protective Equipment, not required under normal use.

Environmental: Refer to specific Member State legislation for requirements under Community environmental legislation.

Section 9: Physical and chemical properties

9.1. Information on basic physical and chemical properties

State: Solid Colour: Various Odour: Characteristic

Evaporation rate:   No data available.

Oxidising:   Non-oxidising (by EC criteria)

Solubility in water:  Miscible

Boiling point/range°C:   Not applicable.		Melting point/range°C:   No data available. Flammability limits %: lower:   No data available.			upper:   No data available. Flash point°C:   Not applicable.	Part.coeff. n-octanol/water:   No data available.

Autoflammability°C:   No data available.	Vapour pressure:   No data available.

Relative density:   No data available.	pH:   Not applicable.

VOC g/l:   No data available.

9.2. Other information

Other information:   In accordance with EU Directive 1272/2008/EC Annex VI (2.2.5) this product will not sustain combustion and is not regarded as requiring a flammable warning.

Section 10: Stability and reactivity

10.1. Reactivity

Reactivity:   Stable under recommended transport or storage conditions.

10.2. Chemical stability

Chemical stability:   Stable under normal conditions. Stable at room temperature.

10.3. Possibility of hazardous reactions

Hazardous reactions: Hazardous reactions will not occur under normal transport or storage conditions.

10.4. Conditions to avoid

# Conditions to avoid:  Heat.  Hot surfaces. Sources of ignition or flames.

10.5. Incompatible materials

Materials to avoid:   Strong oxidising agents. Strong acids.

10.6. Hazardous decomposition products

Haz. decomp. products:   In combustion emits toxic fumes.

Section 11: Toxicological information

11.1. Information on toxicological effects

| Hazardous ingredients: PROPAN-2-OL ORAL | MUS | LD50 | 3600 | mg/kg |
| --- | --- | --- | --- | --- |
| ORAL | RAT | LD50 | 5045 | mg/kg |

POLYMERIC BIGUANIDE HYDROCHLORIDE

| ORAL | RAT | LD50 | >2000 | mg/kg |
| --- | --- | --- | --- | --- |

Toxicity values:   No data available.

Symptoms / routes of exposure

Skin contact:   There may be irritation and redness at the site of contact.

Eye contact: There may be pain and redness. The eyes may water profusely.

Ingestion:   There may be irritation of the throat

Inhalation:   There may be irritation of the throat with a feeling of tightness in the chest.

Delayed / immediate effects:   Not applicable

Section 12: Ecological information

12.1. Toxicity

Ecotoxicity:  No data available

12.2. Persistence and degradability

Persistence and degradability:   No data available

12.3. Bioaccumulative potential

Bioaccumulative potential:   No bioaccumulation potential.

12.4. Mobility in soil

Mobility:   Readily absorbed into spoil

12.5. Results of PBT and vPvB assessment

PBT identification:   This product is not identified as a PBT/vPvB substance.

12.6. Other adverse effects

Other adverse effects:   Negligible ecotoxicity.

Section 13: Disposal considerations

13.1. Waste treatment methods

Disposal operations:   Dispose in to or onto land (eg landfill)

Disposal of packaging:   Dispose of in a regulated landfill site or normal industrial waste

NB: The user's attention is drawn to the possible existence of regional or national regulations regarding disposal.

Section 14: Transport information

Transport class:   This product does not require a classification for transport.

Section 15: Regulatory information

15.1. Safety, health and environmental regulations/legislation specific for the substance or mixture

Specific regulations:   Not applicable.

15.2. Chemical Safety Assessment

Chemical safety assessment: A chemical safety assessment has not been carried out for the substance or the mixture by the supplier.

Section 16: Other information

Other information

Other information: This safety data sheet is prepared in accordance with Commission Regulation (EU) No 453/2010.

This safety data sheet is prepared in accordance with Commission Regulation (EC) No 1272/2008.

Phrases used in s.2 and s.3:   H225: Highly flammable liquid and vapour.

H302: Harmful if swallowed.

H312: Harmful in contact with skin.

H314: Causes severe skin burns and eye damage.

H319: Causes serious eye irritation.

H336: May cause drowsiness or dizziness.

.

Legend to abbreviations:   PNEC = predicted no effect level

DNEL = derived no effect level LD50 = median lethal dose

LC50 = median lethal concentration EC50 = median effective concentration

IC50 = median inhibitory concentration

Dw – dry weight

Bw – body weight

Cc:  closed cup

Oc:  open cup

MUS:  mouse

GPG: Guinea Pig

RBT:  Rabbit

HAM:  Hamster

HMN:  Human

PGN:  Pigeon

IVN:  Intravenous

SCU = subcutaneous SKN = skin

DRM = dermal

OCC = ocular/corneal

PCP = phycico-chemical properties

Legal disclaimer: The above information is believed to be correct but does not purport to be all inclusive and shall be used only as a guide. This company shall not be held liable for any damage resulting from handling or from contact with the above product.

[final page]